# SIMATS ENGINEERING

Department of Computer Science and Engineering

## ASSESSMENT TOOL 2 – DEBUGGING & OPTIMISATION TASK

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO2 – Precision (BL3) | Assessment: | Debugging and Optimisation task |
| Weightage: | 45% | Total Marks: | 100 |
| CO2 Statement: | Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/non-restoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems. |  |  |
| Student Name: |  | Reg. No.: |  |
| Section / Batch: |  | Date: |  |

Instructions to Students

- Identify arithmetic errors and performance bottlenecks in the given problem.

- Analyse the execution steps to locate the source of errors.

- Debug the algorithm by correcting logical and computational faults.

- Optimize the solution using appropriate arithmetic techniques or algorithms.

- Evaluate the optimized solution for correctness, accuracy, and efficiency.

QUESTIONS

- A computer system performing signed binary subtraction using 2's complement produces incorrect results for certain operand combinations. Debug the subtraction process by examining binary conversion, complement generation, and overflow detection. Suggest appropriate corrections to ensure accurate signed arithmetic.

- A digital processor implementing a carry look-ahead adder fails to produce the correct sum for some input combinations. Debug the generate and propagate logic to identify the source of the error. Recommend modifications that restore correct operation while maintaining high-speed performance.

- A hardware multiplier using Booth's algorithm consumes more clock cycles than expected during signed multiplication. Debug the execution sequence by analyzing bit-pair decisions, arithmetic shifts, and register updates. Propose optimization techniques to improve execution efficiency without affecting accuracy.

- A processor implementing the non-restoring division algorithm produces incorrect quotient values for specific dividend and divisor combinations. Debug the step-by-step division process by examining partial remainders, shifting operations, and quotient-bit generation. Suggest corrections to obtain accurate division results.

- A graphics processing application experiences performance degradation due to frequent floating-point computations using IEEE 754 double-precision representation. Debug the arithmetic operations to identify unnecessary precision or computational overhead. Recommend suitable optimization techniques to improve execution speed while maintaining the required numerical accuracy.

Code of Conduct

I 	(Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action.

## Signature of the Student:

RUBRICS FOR CALCULATION

| Criteria | Weightage(%) | Level 4 – Exemplary (5 Marks) | Level 3 – Proficient (4 Marks) | Level 2 – Developing (2–3 Marks) | Level 1 – Beginning (0–1 Mark) |
| --- | --- | --- | --- | --- | --- |
| Identification of Errors / Bugs | 20 | Accurately identifies all errors and issues with complete understanding of the problem | Identifies most errors with minor omissions | Identifies limited errors; misses key issues | Unable to identify errors or misunderstands the problem |
| Debugging Approach & Analysis | 20 | Uses effective debugging techniques with clear analysis and root cause identification | Applies suitable debugging methods with minor gaps | Uses basic debugging methods with limited analysis | No systematic debugging approach followed |
| Correctness of Debugged Solution | 25 | Provides completely corrected solution with accurate functionality and expected output | Provides working solution with minor errors | Partially correct solution with limited functionality | Incorrect solution or fails to resolve errors |
| Optimization Techniques Applied | 20 | Applies efficient optimization methods to improve performance, memory usage and quality | Performs optimization with noticeable improvements | Limited optimization with minimal improvements | No optimization applied or inefficient implementation |
| Testing and Documentation | 15 | Performs complete testing with proper validation and clear documentation | Provides adequate testing and documentation with minor issues | Limited testing and incomplete documentation | No proper testing or documentation provided |

## Marks Evaluation:

| Q. No | Identification of Errors / Bugs (20) |  | Debugging Approach & Analysis (20) | Correctness of Debugged Solution (25) | Optimization Techniques Applied (20) | Testing and Documentation (15) | Total (out of 100) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 2 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 3 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 4 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 5 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
|  | / 20 |  | / 20 | / 25 | / 20 | / 15 | / 100 |

Course Coordinator	Faculty Incharge

- A computer system performing signed binary subtraction using 2's complement produces incorrect results for certain operand combinations. Debug the subtraction process by examining binary conversion, complement generation, and overflow detection. Suggest appropriate corrections to ensure accurate signed arithmetic.

Problem Identification

The computer system produces incorrect results while performing signed binary subtraction using the 2's complement method. The errors mainly occur due to incorrect binary conversion, improper generation of the 2's complement, incorrect carry handling, and failure to detect arithmetic overflow. These mistakes lead to wrong subtraction results, especially when positive and negative numbers are involved.

Debugging Analysis

The subtraction process should first convert both operands into binary form. Instead of direct subtraction, the processor converts the subtrahend into its 2's complement by inverting all bits and adding one. This value is then added to the minuend.

The debugging process includes:

- Verify that both operands use the same number of bits.

- Check whether the 2's complement is generated correctly.

- Ensure the binary adder performs correct addition.

- Ignore the final carry-out only for signed arithmetic.

- Detect overflow by comparing the carry into and carry out of the sign bit.

Overflow occurs when:

- Positive + Positive = Negative

- Negative + Negative = Positive

Corrected Solution

The corrected subtraction process is:

- Convert operands into equal-length binary numbers.

- Generate the correct 2's complement of the subtrahend.

- Add the minuend and complemented subtrahend.

- Ignore the final carry.

- Check overflow using sign-bit comparison.

A = 0101 (+5)

B = 0011 (+3)

2's Complement of B

0011

1100

+0001

-----

1101

0101

1101

----- 0010 (+2)

Optimization Techniques

- Use a dedicated 2's complement generator.

- Perform subtraction using a high-speed Carry Look-Ahead Adder.

- Implement hardware overflow detection.

- Standardize operand length before computation.

Testing and Validation

Test using:

- Positive – Positive

- Positive – Negative

- Negative – Positive

- Negative – Negative

- Overflow conditions

All test cases should produce accurate signed arithmetic results.

- A digital processor implementing a carry look-ahead adder fails to produce the correct sum for some input combinations. Debug the generate and propagate logic to identify the source of the error. Recommend modifications that restore correct operation while maintaining high-speed performance.

Problem Identification

The Carry Look-Ahead Adder generates incorrect sums because of errors in the Generate (G) and Propagate (P) logic. Incorrect carry generation affects the final sum bits and reduces the reliability of the adder.

Debugging Analysis

A Carry Look-Ahead Adder computes carries using:

- Generate:
G = A × B

- Propagate:
P = A ⊕ B

Carry equations:

C1 = G0 + P0C0

C2 = G1 + P1G0 + P1P0C0

Possible debugging steps:

- Verify XOR gates for Propagate.

- Verify AND gates for Generate.

- Check carry equations.

- Validate each carry output.

- Compare outputs with a ripple-carry adder.

Corrected Solution

Replace faulty logic gates and implement the correct carry equations.

Example:

A = 1101

B = 1010

Correct Carry Generation

Correct Sum = 10111

The corrected CLA produces the expected output while maintaining high speed.

Optimization Techniques

- Use hierarchical Carry Look-Ahead blocks.

- Reduce gate delay.

- Minimize logic levels.

- Use parallel carry generation.

Testing and Validation

Verify:

- Random input combinations.

- All carry propagation cases.

- Maximum carry generation.

- Timing analysis.

- Hardware simulation.

The corrected CLA should produce accurate sums with reduced propagation delay.

- A hardware multiplier using Booth's algorithm consumes more clock cycles than expected during signed multiplication. Debug the execution sequence by analyzing bit-pair decisions, arithmetic shifts, and register updates. Propose optimization techniques to improve execution efficiency without affecting accuracy.

### Problem Identification

The Booth multiplier requires more clock cycles than expected. The delay is caused by incorrect bit-pair decisions, unnecessary arithmetic operations, improper arithmetic shifts, and incorrect register updates.

### Debugging Analysis

Booth's Algorithm examines two bits (Q₀ and Q₋₁).

Decision Table:

00 → No operation

11 → No operation

01 → Add Multiplicand

10 → Subtract Multiplicand\

Debugging includes:

- Verify bit-pair checking.

- Confirm correct addition/subtraction.

- Check arithmetic right shifts.

- Verify accumulator and multiplier register updates.

- Ensure loop count equals operand bit length.

Corrected Solution

The corrected algorithm:

- Initialize registers.

- Check Q₀ and Q₋₁.

- Perform addition or subtraction.

- Execute arithmetic right shift.

- Repeat for n iterations.

This removes unnecessary operations and improves execution speed.

Optimization Techniques

- Skip consecutive identical bits.

- Use pipelined multiplication.

- Parallel register shifting.

- Hardware Booth encoder.

These techniques reduce clock cycles without affecting multiplication accuracy.

Testing and Validation

Test using:

- Positive × Positive

- Positive × Negative

- Negative × Negative

- Zero multiplication

- Maximum signed values

All multiplication results should match expected signed arithmetic.

- A processor implementing the non-restoring division algorithm produces incorrect quotient values for specific dividend and divisor combinations. Debug the step-by-step division process by examining partial remainders, shifting operations, and quotient-bit generation. Suggest corrections to obtain accurate division results.

Problem Identification

The processor generates incorrect quotient values because of errors in partial remainder calculation, shifting operations, and quotient-bit assignment.

Debugging Analysis

The Non-Restoring Division algorithm performs repeated shifts and additions/subtractions.

Debugging steps:

- Verify initialization.

- Check left-shift operations.

- Confirm subtraction when remainder is positive.

- Confirm addition when remainder is negative.

- Verify quotient-bit generation.

- Restore the remainder after completion if negative.

Corrected Solution

Correct procedure:

- Initialize Dividend, Divisor and Remainder.

- Shift left.

- If remainder ≥ 0 subtract divisor.

- Otherwise add divisor.

- Set quotient bit.

- Repeat until completion.

- Restore negative remainder.

This produces the correct quotient and remainder.

Optimization Techniques

- Parallel shift registers.

- Fast comparator circuits.

- Hardware control logic.

- Reduce unnecessary restoration operations.

These optimizations improve processor performance.

Testing and Validation

Test cases:

- Exact division

- Division with remainder

- Positive operands

- Negative operands

- Large dividend values

The optimized algorithm should produce correct quotient and remainder for all valid inputs.

- A graphics processing application experiences performance degradation due to frequent floating-point computations using IEEE 754 double-precision representation. Debug the arithmetic operations to identify unnecessary precision or computational overhead. Recommend suitable optimization techniques to improve execution speed while maintaining the required numerical accuracy.

Problem Identification

The graphics processing application experiences slow performance because every computation uses IEEE 754 double-precision arithmetic, even when high precision is unnecessary. This increases computation time and memory usage.

Debugging Analysis

IEEE 754 double precision uses:

- 1 Sign bit

- 11 Exponent bits

- 52 Mantissa bits

Performance issues arise from:

- Unnecessary double-precision calculations.

- Frequent floating-point conversions.

- Repeated arithmetic operations.

- Memory overhead.

Debugging involves:

- Identifying computations requiring only single precision.

- Reducing redundant calculations.

- Checking compiler optimization.

- Profiling execution time.

Corrected Solution

Improve performance by:

- Using single precision wherever acceptable.

- Eliminating unnecessary type conversions.

- Reusing previously computed values.

- Applying compiler optimization techniques.

- Utilizing SIMD/vector instructions.

These changes maintain numerical accuracy while significantly improving execution speed.

Optimization Techniques

- Replace double precision with float where suitable.

- Enable hardware floating-point units.

- Apply loop optimization.

- Cache repeated computations.

- Use GPU parallel processing.

Testing and Validation

Validate using:

- Numerical accuracy tests.

- Floating-point comparison.

- Execution time measurement.

- Memory usage analysis.

- Graphics rendering quality.

The optimized application should execute faster while preserving acceptable numerical precision.