# SIMATS ENGINEERING

Department of Computer Science and Engineering

## ASSESSMENT TOOL 1 – EXPERIMENTS

| Course Code: | CSA1220 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO3 – Precision (BL3) | Assessment: | Experiments |
| Weightage: | 45% | Total Marks: | 100 |
| CO3 Statement: | Design processor organization by constructing pipelined datapath structures, identifying hazard types (data, control, structural), and comparing superscalar techniques such as out-of-order execution, speculative execution, and simultaneous multithreading (SMT). |  |  |
| Student Name: | Sanjay kumar p | Reg. No.: | 192565073 |
| Section / Batch: | Slot D | Date: | 10/08/2026 |

Instructions to Students

- Identify the relevant concepts of register transfers, ALU operations, bus organization, pipelining, hazards, and superscalar processing.

- Analyze the execution of data transfers, arithmetic and logic operations, instruction processing, and parallel execution.

- Apply suitable techniques such as multiple buses, pipelining, stalling, forwarding, branch prediction, and speculative execution.

- Compute performance measures such as execution time, clock cycles, speedup, throughput, and resource utilization.

- Interpret the results by evaluating performance improvements, bottlenecks, and the suitability of each architectural technique.

QUESTIONS

- Analyze a processor datapath that performs register transfers and logical operations such as XOR, NOT, and comparison, and examine the control signals and execution sequence required for each operation.

- Develop a bus-based system for transferring data between the processor, memory, and I/O devices, and analyze how bus width, transfer rate, and bus contention influence system performance.

- Simulate a pipelined processor with arithmetic, memory, and branch instructions, and evaluate the effects of pipeline depth on execution time, throughput, and overall speedup.

- Construct a pipeline containing instruction dependencies and branch conditions, and analyze the performance impact of data and control hazards using stalling, forwarding, and branch prediction techniques.

- Develop a superscalar processor model that supports multiple execution units and out-of-order instruction processing, and evaluate how instruction scheduling and resource allocation improve instruction throughput.

Code of Conduct

I sanjay kumar p(192565073)	(Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action.

## Signature of the Student: sanjay kumar p

RUBRICS FOR CALCULATION

| Criteria | Weightage(%) | Level 4 – Exemplary (5 Marks) | Level 3 – Proficient (4 Marks) | Level 2 – Developing (2–3 Marks) | Level 1 – Beginning (0–1 Mark) |
| --- | --- | --- | --- | --- | --- |
| Understanding of Concepts | 25 | Demonstrates thorough understanding and effectively integrates multiple concepts/technologies | Good understanding with proper integration of most concepts | Basic understanding; limited or partial integration | Poor understanding; unable to integrate concepts |
| Experimental Design | 30 | Well-planned, systematic implementation with optimal solution | Correct implementation with minor issues | Basic implementation with errors or inefficiencies | Incorrect or incomplete implementation |
| Use of Tools | 20 | Effective and advanced use of tools/technologies with proper configuration | Appropriate use of tools with correct execution | Limited or inconsistent use of tools | Incorrect or minimal use of tools |
| Analysis of Results | 15 | Insightful analysis with accurate interpretation and validation of results | Good analysis with reasonable interpretation | Basic analysis with limited insights | Poor or incorrect analysis of results |
| Documentation | 10 | Clear, well-structured documentation with diagrams, code, and results | Organized documentation with necessary details | Basic documentation with missing details | Poor or incomplete documentation |

## Marks Evaluation:

| Q. No | Understanding of Concepts(25) |  | Experimental Design (30) | Use of Tools(20) | Analysis of Results (15) | Documentation (10) | Total (out of 100) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 2 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 3 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 4 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 5 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
|  | / 25 |  | / 30 | / 20 | / 15 | / 10 | / 100 |

Course Coordinator	Faculty Incharge

Q1. Processor Datapath – Register Transfers and Logical Operations

A processor datapath is the part of the CPU responsible for transferring, storing, and processing data. It mainly consists of registers, an ALU, buses, multiplexers, and control logic.

Basic Datapath

Register R1/R2 → MUX → ALU (XOR / NOT / Compare) → Result Register R3

The control unit generates signals that select source registers, choose the ALU operation, enable the ALU output, and load the destination register.

1. Register Transfer

For the operation R2 ← R1:

Select R1 as the source register.

Enable R1 output so its contents are placed on the internal bus.

Enable the input/load control of R2.

On the active clock edge, R2 stores the value from R1.

| Control Signal | Function |
| --- | --- |
| R1out | Places R1 contents on the bus |
| R2in | Loads the bus value into R2 |
| Clock | Synchronizes the transfer |

2. XOR Operation

For R3 ← R1 XOR R2, the ALU receives R1 and R2 as inputs and performs a bitwise exclusive-OR operation.

Example: R1 = 1010, R2 = 1100. Therefore, R1 XOR R2 = 0110.

Select R1 and R2 as ALU inputs.

Set the ALU control lines to XOR.

Enable the ALU output.

Enable R3 input to store the result.

3. NOT Operation

For R2 ← NOT(R1), the ALU inverts every bit of R1. Example: R1 = 1010, so NOT R1 = 0101.

Typical control signals are R1out, ALU_NOT, ALUout, R2in, and Clock.

4. Comparison Operation

For CMP R1,R2, the ALU generally performs R1 − R2 and updates status flags rather than storing the arithmetic result.

Zero flag (Z)

Carry flag (C)

Sign/Negative flag (S/N)

Overflow flag (V)

For example, if R1 = 10 and R2 = 10, then R1 − R2 = 0 and the Zero Flag becomes 1, indicating equality.

Conclusion: The datapath performs transfers and logical operations through coordinated control signals. The ALU performs XOR, NOT, and comparison, while registers store intermediate and final results.

Q2. Bus-Based System for CPU, Memory and I/O

A bus is a communication pathway used to transfer information between the processor, memory, and I/O devices.

Basic Bus Organization

CPU ↔ SYSTEM BUS (Address Bus + Data Bus + Control Bus) ↔ Memory / I/O Devices

Types of Buses

| Bus | Purpose |
| --- | --- |
| Address Bus | Carries the address of a memory location or I/O device |
| Data Bus | Carries the actual data being transferred |
| Control Bus | Carries Read, Write, Interrupt, Bus Request and Bus Acknowledge signals |

Effect of Bus Width

A wider data bus transfers more bits in a single transfer. The theoretical bandwidth can be expressed as:

Bandwidth = Bus Width × Transfer Rate

Example: A 32-bit bus operating at 100 MHz provides 32 × 100 million = 3.2 Gbit/s, which is approximately 400 MB/s, assuming one transfer per clock cycle.

Effect of Transfer Rate

Increasing the bus frequency increases the number of transfers per second. For example, a 200 MHz bus can theoretically provide twice the transfer capacity of a 100 MHz bus when both buses have the same width.

Bus Contention

Bus contention occurs when multiple devices attempt to use the same shared bus at the same time. Arbitration is required to decide which device gets access.

Waiting time and latency increase.

Throughput decreases.

CPU or I/O devices may experience stalls.

System performance can be reduced.

Methods to Reduce Contention

Bus arbitration

Multiple buses

Separate memory and I/O buses

Direct Memory Access (DMA)

Bus buffering

Conclusion: Bus width and transfer rate determine data-transfer capacity, while contention can reduce performance. An efficient bus system therefore requires adequate width, high transfer rate, and effective arbitration.

Q3. Pipelined Processor with Arithmetic, Memory and Branch Instructions

Pipelining divides instruction execution into stages so that several instructions can be processed simultaneously. A typical five-stage pipeline is IF → ID → EX → MEM → WB.

| Stage | Function |
| --- | --- |
| IF | Instruction Fetch |
| ID | Instruction Decode |
| EX | Execute / ALU operation |
| MEM | Memory Access |
| WB | Write Back |

Example instruction sequence: I1: ADD R1,R2,R3; I2: SUB R4,R5,R6; I3: LOAD R7,0(R8); I4: ADD R9,R10,R11; I5: BEQ R1,R4,LABEL.

| Cycle | I1 | I2 | I3 | I4 | I5 |
| --- | --- | --- | --- | --- | --- |
| 1 | IF |  |  |  |  |
| 2 | ID | IF |  |  |  |
| 3 | EX | ID | IF |  |  |
| 4 | MEM | EX | ID | IF |  |
| 5 | WB | MEM | EX | ID | IF |
| 6 |  | WB | MEM | EX | ID |
| 7 |  |  | WB | MEM | EX |
| 8 |  |  |  | WB | MEM |
| 9 |  |  |  |  | WB |

Cycle and Speedup Calculation

For k pipeline stages and n instructions, ideal execution cycles are approximately: Cycles = k + n − 1.

For k = 5 and n = 5: Cycles = 5 + 5 − 1 = 9 cycles.

Without pipelining, assuming five cycles per instruction: 5 × 5 = 25 cycles.

Ideal speedup = 25 / 9 ≈ 2.78.

Effect of Pipeline Depth

Deeper pipelines can permit a higher clock frequency.

Throughput can increase because more instructions overlap.

Very deep pipelines increase branch penalties and pipeline-register overhead.

Hazards and pipeline flushing can reduce the practical speedup.

Conclusion: Pipelining improves instruction throughput by overlapping execution, but deeper pipelines do not always give proportional performance gains because of hazards, branch penalties, and overhead.

Q4. Pipeline Hazards – Stalling, Forwarding and Branch Prediction

A pipeline hazard is a condition that prevents an instruction from executing in its intended clock cycle. The main hazards are data, control, and structural hazards.

Data Hazard

Consider: I1: ADD R1,R2,R3 and I2: SUB R4,R1,R5. Instruction I2 depends on the value of R1 produced by I1. This is a Read After Write (RAW) dependency.

Stalling

The processor temporarily delays the dependent instruction until the required data becomes available. A stall introduces one or more idle cycles and therefore reduces throughput.

Forwarding

Forwarding sends a result directly from an earlier pipeline stage to a later stage without waiting for the result to be written to the register file. This reduces unnecessary stalls and improves performance.

Control Hazard

A control hazard occurs with branch instructions because the processor may not immediately know the correct next instruction.

Example: I1: BEQ R1,R2,LABEL. If the branch is taken, instructions fetched after the branch may be incorrect and must be discarded.

Branch Prediction

The processor predicts whether a branch will be taken or not taken. A correct prediction allows the pipeline to continue with little disruption; an incorrect prediction causes a pipeline flush and a performance penalty.

Example: If 100 branch instructions occur and prediction accuracy is 90%, then 10 branches are mispredicted. If each misprediction costs 3 cycles, the penalty is 10 × 3 = 30 cycles.

| Technique | Advantage | Disadvantage |
| --- | --- | --- |
| Stalling | Simple and reliable | Reduces throughput |
| Forwarding | Reduces data stalls | Requires additional hardware |
| Branch Prediction | Reduces control-hazard penalty | Wrong predictions cause pipeline flushes |

Conclusion: Forwarding and stalling handle data dependencies, while branch prediction reduces control-hazard delays. Combining these techniques provides better pipeline performance.

Q5. Superscalar Processor with Multiple Execution Units and Out-of-Order Execution

A superscalar processor can execute more than one instruction per clock cycle by using multiple execution units.

Basic Superscalar Organization

Instruction Fetch → Instruction Scheduler → Multiple Execution Units → Write Back

| Execution Unit | Typical Operation |
| --- | --- |
| ALU 1 | Integer arithmetic |
| ALU 2 | Integer and logic operations |
| FPU | Floating-point operations |
| Load/Store Unit | Memory operations |

In-Order vs Out-of-Order Execution

In-order execution follows program order. If an early instruction is delayed, later instructions may also wait.

Out-of-order execution allows independent instructions to execute before an earlier instruction that is waiting.

Example: I1 = LOAD R1,[A], I2 = ADD R4,R5,R6, and I3 = SUB R7,R8,R9. If I1 is waiting for memory, I2 and I3 can execute because they are independent.

Instruction Scheduling

| Instruction | Dependency | Status |
| --- | --- | --- |
| I1: LOAD | Memory | Waiting |
| I2: ADD | None | Ready |
| I3: SUB | None | Ready |
| I4: ADD | I1 | Waiting |

The scheduler can issue I2 and I3 while I1 waits, keeping execution units busy. Once I1 completes, I4 can execute.

Resource Allocation

The scheduler allocates instructions according to the available execution units. For example, with two ALUs, one FPU, and one Load/Store Unit, independent ADD, SUB, floating-point, and LOAD instructions can be distributed across the units.

Throughput and Speedup

If a processor can issue two instructions per cycle, its ideal IPC (Instructions Per Cycle) is 2. For 10 independent instructions, ideal execution requires approximately 10/2 = 5 cycles, compared with about 10 cycles for a single-issue processor.

Ideal speedup ≈ 10/5 = 2.

Actual speedup is normally lower because of dependencies, branches, cache misses, memory latency, and limited execution resources.

Conclusion: Superscalar processors increase instruction throughput by executing multiple instructions simultaneously. Out-of-order execution, instruction scheduling, and efficient resource allocation keep execution units busy and improve performance.

Overall Conclusion

The five experiments demonstrate how modern processors improve performance through datapath organization, efficient bus communication, pipelining, hazard handling, and superscalar execution. Register transfers and ALU operations provide the basic processing mechanism; buses provide communication; pipelining increases instruction overlap; forwarding, stalling, and branch prediction reduce hazards; and superscalar out-of-order execution exploits instruction-level parallelism. Together, these techniques improve execution time, throughput, speedup, and resource utilization.