# SIMATS ENGINEERING

Department of Computer Science and Engineering

## ASSESSMENT TOOL 2 – COMPARATIVE ANALYSIS

| Course Code: | CSA1220 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO4 – Precision (BL4) | Assessment: | Comparative Analysis |
| Weightage: | 30% | Total Marks: | 100 |
| CO4 Statement: | Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics. |  |  |
| Student Name: | Sanjay kumar P | Reg. No.: | 192565073 |
| Section / Batch: | Slot B-CSA1220 | Date: | 24-08-2026 |

Instructions to Students

- Identify the relevant concepts of L1, L2, and L3 cache, SRAM, DRAM, NAND Flash, MRAM, RRAM, virtual memory, paging, latency, bandwidth, and throughput.

- Analyze the memory requirements of different computer systems by considering cache levels, memory technologies, access latency, bandwidth, throughput, capacity, and energy consumption.

- Evaluate the performance characteristics of different memory technologies and determine their suitability for cache memory, main memory, secondary storage, and future hierarchical memory systems.

- Compute relevant performance measures such as access time, latency, bandwidth, throughput, hit/miss rate, average memory access time (AMAT), speedup, and energy efficiency wherever applicable.

- Interpret the results by assessing memory bottlenecks, performance improvements, technology trade-offs, and the suitability of each memory technology or memory hierarchy for a given application.

QUESTIONS

- Analyze the relationship between cache size, cache access time, and cache hit rate. Evaluate how changes in cache size affect processor performance, memory latency, and overall system throughput.

- Assess the impact of memory bandwidth on data-intensive applications such as artificial intelligence, scientific computing, and multimedia processing. Analyze how insufficient bandwidth can create memory bottlenecks and limit processor performance.

- Evaluate the effect of page size on virtual memory performance. Analyze how page size influences page faults, memory utilization, translation overhead, and system throughput.

- Examine the role of data locality in improving cache performance. Analyze how temporal and spatial locality influence cache hit rates, memory access latency, and processor efficiency.

- Assess the suitability of SRAM, DRAM, NAND Flash, MRAM, and RRAM for different levels of a memory hierarchy. Evaluate each technology based on latency, bandwidth, capacity, energy consumption, endurance, and cost, and recommend appropriate technologies for specific applications.

Code of Conduct

I 	(Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action.

## Signature of the Student:

RUBRICS FOR CALCULATION

| Criteria | Weightage(%) | Level 4 – Exemplary (5 Marks) | Level 3 – Proficient (4 Marks) | Level 2 – Developing (2–3 Marks) | Level 1 – Beginning (0–1 Mark) |
| --- | --- | --- | --- | --- | --- |
| Scope of Items | 20% | Selects highly relevant items with clear scope and justification | Selects appropriate items with minor gaps | Selection is limited or partially relevant | Items are unclear or not relevant |
| Comparison Depth | 25% | Provides detailed and comprehensive comparison across multiple aspects | Provides adequate comparison with some depth | Limited comparison with few aspects | Very minimal or no comparison |
| Use of Evidence | 20% | Supports comparison with accurate data, examples, or references | Uses some supporting evidence with minor gaps | Limited or weak supporting evidence | No supporting evidence provided |
| Critical Thinking | 20% | Demonstrates strong critical thinking with clear interpretation and reasoning | Shows reasonable interpretation with minor gaps | Basic interpretation; lacks depth | No meaningful interpretation |
| Clarity of Presentation | 15% | Well-organized, clear, and logically structured presentation | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganized and difficult to understand |

## Marks Evaluation:

| Q. No | Scope of Items (20) |  | Comparison Depth (25) |  | Use of Evidence (20) | Critical Thinking (20) | Clarity of Presentation (15) | Total (out of 100) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
| 2 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
| 3 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
| 4 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
| 5 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
|  | / 20 |  | / 25 |  | / 20 | / 20 | / 15 | / 100 |

Course Coordinator	Faculty Incharge

- Analyze the relationship between cache size, cache access time, and cache hit rate. Evaluate how changes in cache size affect processor performance, memory latency, and overall system throughput.

Cache memory is a small and high-speed memory located close to the processor. Modern processors generally use multiple cache levels such as L1, L2, and L3. The main purpose of cache is to reduce the time required to access frequently used data and instructions.

Cache size and hit rate:

Increasing cache size generally increases the probability that required data will be present in the cache. Therefore, the cache hit rate increases and the miss rate decreases. For example, a small cache may frequently replace data, causing more cache misses, while a larger cache can retain more frequently accessed information.

Cache size and access time:

A larger cache can require more hardware for searching and managing cache entries. Therefore, increasing cache size may increase its access time. This is one reason processors use a hierarchy of caches rather than one extremely large cache.

- L1 cache: Smallest and fastest; usually accessed first.

- L2 cache: Larger than L1 but slower.

- L3 cache: Larger and slower than L2, often shared among processor cores.

The relationship can be represented using Average Memory Access Time (AMAT):

AMAT = Hit Time + (Miss Rate × Miss Penalty)

For example, assume:

- L1 hit time = 1 ns

- L1 miss rate = 5%

- Main-memory miss penalty = 80 ns

Then:

AMAT = 1 + (0.05 × 80),AMAT = 5 ns

If increasing the cache size reduces the miss rate to 2% while keeping the hit time at 1 ns:

AMAT = 1 + (0.02 × 80),AMAT = 2.6 ns

Thus, the larger cache improves average memory access time in this example.

However, continuously increasing cache size is not always beneficial. A very large cache can increase hardware cost, power consumption, and access time. Therefore, computer architects select cache sizes by balancing capacity, latency, hit rate, power, and cost.

Effect on processor performance:

A higher hit rate means the processor spends less time waiting for slower memory. This reduces memory stalls and increases processor efficiency and overall system throughput. Therefore, an appropriately sized cache can significantly improve performance, but the optimum size depends on the workload and processor architecture.

- Assess the impact of memory bandwidth on data-intensive applications such as artificial intelligence, scientific computing, and multimedia processing. Analyze how insufficient bandwidth can create memory bottlenecks and limit processor performance.

Memory bandwidth is the amount of data that can be transferred between memory and the processor or other components per unit of time. It is commonly expressed in GB/s.

Data-intensive applications require large amounts of data to be transferred continuously.

Examples include:

- Artificial Intelligence and Machine Learning

- Scientific computing

- Multimedia processing

- Video processing

- Large-scale simulations

Artificial Intelligence

AI workloads frequently process large matrices, tensors, model parameters, and training datasets. GPUs and CPUs can perform calculations very quickly, but they also need to continuously fetch data from memory.

If the processor can perform calculations faster than the memory system can supply data, the processor becomes idle while waiting for memory. This condition is known as a memory bottleneck.

For example, assume an application needs to transfer 500 GB of data and the memory system provides 100 GB/s bandwidth.

Transfer time = Data size / Bandwidth

= 500 / 100 = 5 seconds

If the bandwidth is increased to 200 GB/s:

Transfer time = 500 / 200 = 2.5 seconds

Therefore, higher bandwidth can significantly reduce data-transfer time.

Scientific Computing

Scientific applications such as weather simulation, molecular modelling, physics simulations, and numerical calculations process large datasets. Many of these workloads repeatedly read and write arrays of data. Insufficient memory bandwidth can prevent the processor from reaching its maximum computational performance.

Multimedia Processing

Video editing, image processing, 3D rendering, and high-resolution video applications continuously move large amounts of image and video data. Higher memory bandwidth helps process these large streams more efficiently.

Memory Bottleneck:

A simplified performance relationship is:

Effective performance ≈ Minimum of Compute Capability and Memory Data Supply

If the processor can process 1 TB/s of data but memory can supply only 200 GB/s, the processor cannot fully utilize its computational capability.

Therefore:

Low bandwidth → processor waiting → memory stalls → lower throughput

High bandwidth → faster data supply → better processor utilization → higher throughput

However, bandwidth alone does not determine performance. Latency, cache hit rate, access pattern, memory capacity, and processor architecture also affect performance.

Conclusion:
High memory bandwidth is particularly important for data-intensive applications because it allows large quantities of data to be supplied to processors quickly. Insufficient bandwidth can become a major bottleneck even when the processor itself is extremely powerful.

- Evaluate the effect of page size on virtual memory performance. Analyze how page size influences page faults, memory utilization, translation overhead, and system throughput.

Virtual memory allows a system to use secondary storage as an extension of physical memory. It divides virtual and physical memory into fixed-size blocks called pages. The operating system maps virtual pages to physical frames using page tables.

Page size has a significant effect on virtual-memory performance.

Small Page Size

Advantages:

- Better memory utilization

- Less unused space inside pages

- More precise allocation

- Potentially lower wasted memory

Disadvantages:

- More pages are required for the same amount of memory

- Larger page tables

- More address-translation overhead

- More page-table entries to manage

Large Page Size

Advantages:

- Fewer pages are required

- Smaller page tables

- Reduced translation overhead

- Better TLB coverage

- Can improve sequential data access

Disadvantages:

- Greater internal fragmentation

- More data may be transferred than actually required

- A process may occupy more physical memory than necessary

- Large pages can be less efficient for scattered memory accesses

For example, suppose a program requires 1 GB of virtual memory.

With a page size of 4 KB:

Number of pages = 1 GB / 4 KB

= 262,144 pages

With a page size of 2 MB:

Number of pages = 1 GB / 2 MB

= 512 pages

Thus, a larger page size greatly reduces the number of page-table entries.

Page Faults

A page fault occurs when a required page is not currently available in physical memory. Page faults are expensive because the operating system may need to retrieve data from secondary storage.

Larger pages can sometimes reduce the number of page faults when programs access memory sequentially because more useful data is brought into memory during one page transfer. However, if the program accesses memory randomly, loading a large page may bring in data that is never used.

Translation Overhead

Virtual addresses must be translated into physical addresses. The Translation Lookaside Buffer (TLB) stores recently used address translations.

A larger page size allows each TLB entry to cover more memory. Therefore, large pages can improve TLB coverage and reduce TLB misses.

Overall Effect

| Factor | Small Page | Large Page |
| --- | --- | --- |
| Page-table size | Larger | Smaller |
| Translation overhead | Higher | Lower |
| Internal fragmentation | Lower | Higher |
| TLB coverage | Lower | Higher |
| Memory utilization | Better for small allocations | Can be lower |
| Sequential access | Moderate | Often better |
| Random access | Often better | May waste memory |

Conclusion:
There is no universally optimal page size. Small pages improve memory utilization, while large pages reduce translation overhead and can improve throughput. Modern systems therefore use page-size choices according to workload requirements.

- Examine the role of data locality in improving cache performance. Analyze how temporal and spatial locality influence cache hit rates, memory access latency, and processor efficiency.

Data locality refers to the tendency of programs to access the same or nearby data repeatedly. Cache memory takes advantage of this behavior to reduce memory access latency.

There are two major forms of locality:

- Temporal locality

- Spatial locality

Temporal Locality

Temporal locality means that if a program accesses a particular data item, it is likely to access the same item again soon.

Example:

for(i = 0; i < 1000; i++)

sum = sum + value;

The variable sum is repeatedly accessed. Keeping it in a cache allows subsequent accesses to occur quickly.

Temporal locality improves cache hit rate because recently accessed data remains available in the cache.

Spatial Locality

Spatial locality means that when a particular memory location is accessed, nearby memory locations are likely to be accessed soon.

Example:

for(i = 0; i < 1000; i++)

sum = sum + array[i];

Array elements are stored in consecutive memory locations. When one cache block is loaded, nearby array elements can also be brought into the cache.

Therefore, spatial locality allows one memory transfer to support multiple future accesses.

Effect on Cache Hit Rate

Good locality:

More cache hits → fewer memory accesses → lower latency → better processor efficiency

Poor locality:

More cache misses → more main-memory accesses → higher latency → lower performance

For example, sequentially accessing an array generally has better spatial locality than randomly accessing elements throughout a large memory region.

Effect on Processor Efficiency

When cache hit rates are high, the CPU spends more time executing instructions and less time waiting for main memory. Therefore, good locality improves processor utilization and overall throughput.

The relationship can be summarized as:

Good temporal locality + Good spatial locality
→ High cache hit rate
→ Lower AMAT
→ Fewer CPU stalls
→ Higher processor efficiency

Thus, programmers and compiler designers often organize data structures and algorithms to improve locality.

- Assess the suitability of SRAM, DRAM, NAND Flash, MRAM, and RRAM for different levels of a memory hierarchy. Evaluate each technology based on latency, bandwidth, capacity, energy consumption, endurance, and cost, and recommend appropriate technologies for specific applications.

Different memory technologies have different characteristics. Therefore, no single technology is ideal for every level of the memory hierarchy.

| Technology | Latency | Capacity | Volatility | Endurance | Energy | Typical Application |
| --- | --- | --- | --- | --- | --- | --- |
| SRAM | Very low | Low | Volatile | Very high | Higher per bit | CPU cache |
| DRAM | Low | Medium–High | Volatile | Very high | Moderate | Main memory |
| NAND Flash | Higher | Very high | Non-volatile | Limited writes | Low per bit | SSD/storage |
| MRAM | Low | Medium | Non-volatile | Very high | Low/Moderate | Embedded memory, future cache/main memory |
| RRAM | Potentially low | High potential | Non-volatile | Technology-dependent | Potentially low | Emerging storage/memory |

SRAM

Static Random Access Memory (SRAM) uses flip-flop-based storage and does not require periodic refresh while powered.

Advantages:

- Extremely fast

- Very low access latency

- High endurance

- Suitable for frequent processor accesses

Disadvantages:

- Requires more transistors per bit

- Expensive per bit

- Lower storage density

- Higher area requirement

Recommended application:

SRAM is highly suitable for L1, L2, and parts of L3 cache because cache requires very fast access.

DRAM

Dynamic Random Access Memory (DRAM) stores information using capacitors and requires periodic refreshing.

Advantages:

- Higher density than SRAM

- Lower cost per bit

- Suitable for large memory capacities

- Good performance for main-memory applications

Disadvantages:

- Slower than SRAM

- Requires refresh operations

- Volatile

- Higher access latency than cache memory

Recommended application:

DRAM is the standard choice for main memory/RAM because it provides a good balance between capacity, cost, and performance.

NAND Flash

NAND Flash is a non-volatile memory technology, meaning it retains information without power.

Advantages:

- Very high storage density

- Non-volatile

- Low cost per bit

- Low idle power

- Suitable for large storage capacities

Disadvantages:

- Higher latency than RAM

- Limited program/erase endurance

- Write operations are slower

- Requires controllers and wear-management techniques

Recommended application:

NAND Flash is highly suitable for SSDs, memory cards, USB storage, and other secondary-storage applications.

MRAM

Magnetoresistive Random Access Memory (MRAM) stores information using magnetic states.

Advantages:

- Non-volatile

- Fast access

- High endurance

- No refresh requirement

- Good potential for low-power systems

Disadvantages:

- Higher cost than conventional DRAM in many applications

- Density and manufacturing challenges

- Technology is less widespread than DRAM and NAND Flash

Recommended application:

MRAM is suitable for embedded systems, persistent memory, and applications requiring fast non-volatile storage. It also has potential for future memory hierarchies.

RRAM

Resistive Random Access Memory (RRAM) stores data by changing the resistance of a material.

Advantages:

- Non-volatile

- Potentially high density

- Low power potential

- Fast switching potential

- Suitable for emerging memory architectures

Disadvantages:

- Manufacturing challenges

- Endurance and reliability can vary

- Technology is still developing

- Large-scale adoption is more limited than conventional memory technologies