# SIMATS ENGINEERING

Department of Computer Science and Engineering

## ASSESSMENT TOOL 1 – CONSTRAINT-BASED PROBLEM SOLVING

| Course Code: | CSA1220 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO5 – Precision (BL4) | Assessment: | Constraint Based Problem Solving |
| Weightage: | 40% | Total Marks: | 100 |
| CO5 Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: | Sanjay Kumar P | Reg. No.: | 192565073 |
| Section / Batch: | Slot-B | Date: | 25-08-2026 |

Instructions to Students

- Identify the relevant concepts of I/O interfaces, interrupts, DMA, bus arbitration, and high-speed communication protocols.

- Analyze the I/O requirements considering CPU utilization, latency, throughput, bandwidth, and device priorities.

- Apply suitable I/O techniques such as DMA, vectored interrupts, memory-mapped I/O, nested interrupts, and bus arbitration.

- Compute performance measures such as data transfer rate, interrupt latency, CPU utilization, throughput, and transfer time.

- Interpret the results by evaluating CPU efficiency, communication performance, interrupt response, and suitability of the proposed I/O architecture.

QUESTIONS

- Design an I/O communication mechanism for a smart agriculture system with the following constraints:

  - 50 field sensors transmit readings at different intervals.

  - Soil-moisture alerts must be processed within 20 µs.

  - Use interrupt-driven I/O for critical events.

  - Use DMA for periodic bulk sensor-data transfer.

- Construct an I/O subsystem for a video surveillance system with the following constraints:

  - 8 cameras continuously generate video streams.

  - The aggregate data rate is 1.5 GB/s.

  - Minimize CPU involvement during video transfer.

  - Use DMA with buffer management to prevent data loss.

- Design a peripheral communication architecture for an industrial automation system with the following constraints:

  - 12 peripherals operate at different data rates.

  - High-priority control devices require deterministic response.

  - Low-priority devices must not monopolize the communication bus.

  - Implement priority-based bus arbitration.

- Construct a data transfer mechanism for a real-time audio processing system with the following constraints:

  - Audio data must be transferred continuously at 200 MB/s.

  - Transfer interruptions must be minimized.

  - Use double buffering for continuous data processing.

  - Generate an interrupt only when a buffer transfer is completed.

- Design an interrupt management system for a multicore embedded processor with the following constraints:

  - Support multiple interrupt sources simultaneously.

  - Distribute interrupts across processor cores to balance the workload.

  - Critical interrupts must have higher priority than routine interrupts.

  - Prevent excessive interrupt overhead during high system activity.

Code of Conduct

I sanjay kumar P(192565073)	(Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action.

## Signature of the Student: sanjay kumar P

RUBRICS FOR CALCULATION

| Criteria | Weightage(%) | Level 4 – Exemplary (5 Marks) | Level 3 – Proficient (4 Marks) | Level 2 – Developing (2–3 Marks) | Level 1 – Beginning (0–1 Mark) |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 25% | Clearly defines the problem and identifies all relevant constraints accurately | Identifies most constraints with minor gaps | Partial understanding; misses key constraints | Fails to identify constraints correctly |
| Constraint Analysis | 25% | Analyzes constraints effectively and prioritizes them logically | Provides reasonable analysis with some prioritization | Limited analysis; weak prioritization | No meaningful analysis or prioritization |
| Solution Development | 25% | Develops optimal and feasible solutions satisfying all constraints | Develops workable solutions with minor limitations | Solutions partially satisfy constraints | Solutions do not meet constraints |
| Application of Concepts | 15% | Effectively applies appropriate concepts, methods, or tools | Applies concepts with minor errors | Limited application; requires guidance | Unable to apply relevant concepts |
| Justification | 10% | Provides strong justification for decisions with logical reasoning | Provides justification with some clarity | Weak or unclear justification | No justification provided |

## Marks Evaluation:

| Q. No | Problem Understanding (25) |  | Constraint Analysis (25) |  | Solution Development (25) | Application of Concepts (15) | Justification (10) | Total (out of 100) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
| 2 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
| 3 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
| 4 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
| 5 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
|  | / 25 |  | /25 |  | / 25 | / 15 | / 10 | / 100 |

Course Coordinator	Faculty Incharge

- Design an I/O communication mechanism for a smart agriculture system with the following constraints:

  - 50 field sensors transmit readings at different intervals.

  - Soil-moisture alerts must be processed within 20 µs.

  - Use interrupt-driven I/O for critical events.

  - Use DMA for periodic bulk sensor-data transfer.

The smart agriculture system contains 50 field sensors that transmit readings at different intervals. Soil-moisture alerts are critical and must be processed within 20 µs. Periodic sensor data is comparatively less time-sensitive and can be transferred using DMA.

Constraint Analysis

- 50 sensors generate data at different rates.

- Critical soil-moisture events require very low latency.

- CPU must respond immediately to critical events.

- Periodic bulk data should not continuously occupy the CPU.

- The system should support efficient and reliable data transfer.

Proposed Architecture

50 Field Sensors

|

Sensor Interface

|

+----------+----------+

|                     |

Critical Alerts        Periodic Data

|                     |

Interrupt Controller       DMA Controller

|                     |

v                     v

CPU <------------> Main Memory

|

Agriculture

Control System

Solution

An interrupt-driven I/O mechanism is used for critical soil-moisture alerts. When a sensor detects a dangerous moisture level, it generates an interrupt. The interrupt controller immediately sends the request to the CPU. The CPU executes the appropriate interrupt service routine and processes the alert.

For periodic sensor readings, a DMA controller is used. The DMA controller transfers blocks of sensor data directly between the sensor interface and main memory without requiring the CPU to handle every byte.

The DMA transfer can be organized using buffers:

Sensor Data → DMA → Memory Buffer → CPU Processing

Interrupt Handling

A vectored interrupt can be used so that the CPU directly identifies the appropriate interrupt service routine. High-priority soil-moisture alerts are given higher priority than normal sensor readings.

Justification

This architecture satisfies the 20 µs response requirement by using interrupt-driven I/O for critical events. DMA reduces CPU involvement during bulk transfers and allows the CPU to concentrate on control and decision-making tasks. Therefore, the design provides low latency and efficient sensor-data transfer.

- Construct an I/O subsystem for a video surveillance system with the following constraints:

  - 8 cameras continuously generate video streams.

  - The aggregate data rate is 1.5 GB/s.

  - Minimize CPU involvement during video transfer.

  - Use DMA with buffer management to prevent data loss.

The system contains 8 continuously operating cameras with an aggregate data rate of 1.5 GB/s. CPU involvement must be minimized, and DMA with buffer management must be used to prevent video data loss.

Constraint Analysis

- 8 cameras continuously produce data.

- Total incoming data rate = 1.5 GB/s.

- The CPU should not process every video transfer.

- Continuous data requires high-throughput transfer.

- Buffers are required to avoid overflow and data loss.

Proposed Architecture

Camera 1 ─┐

Camera 2 ─┤

Camera 3 ─┤

Camera 4 ─┤

Camera 5 ─┤

Camera 6 ─┤──> Video Interface ──> DMA Controller

Camera 7 ─┤                              |

Camera 8 ─┘                              v

Buffer Memory

|

v

Main Memory

|

v

CPU / GPU

Processing

Solution

A multi-channel DMA controller can be used to transfer video streams from the camera interface to memory. Each camera can be assigned a DMA channel or logical stream.

The DMA controller transfers data directly into memory, significantly reducing CPU involvement.

A circular or double-buffering scheme can be used:

Camera Data

|

v

+----------+       +----------+

| Buffer A | ----> | Processing|

+----------+       +----------+

^

|

+----------+

| Buffer B |

+----------+

While Buffer A is being processed, DMA fills Buffer B. After Buffer B is filled, DMA switches back to Buffer A.

Data Rate Calculation

Aggregate data rate:

1.5 GB/s = 1500 MB/s

For 8 cameras:

Average rate per camera = 1500 / 8

= 187.5 MB/s per camera

Therefore, each camera produces approximately 187.5 MB/s, assuming equal data rates.

Justification

DMA provides high-speed direct memory transfer while minimizing CPU involvement. Double or circular buffering ensures that one buffer can receive new data while another is being processed. This prevents buffer overflow and reduces the possibility of lost video frames.

The proposed architecture is therefore suitable for continuous 1.5 GB/s surveillance data.

- Design a peripheral communication architecture for an industrial automation system with the following constraints:

  - 12 peripherals operate at different data rates.

  - High-priority control devices require deterministic response.

  - Low-priority devices must not monopolize the communication bus.

  - Implement priority-based bus arbitration.

The industrial automation system has 12 peripherals operating at different data rates. High-priority control devices need deterministic response, while low-priority devices must not monopolize the communication bus. A priority-based bus arbitration mechanism is required.

Constraint Analysis

- 12 peripherals have different communication requirements.

- Control devices require predictable response time.

- Low-priority devices must receive bus access fairly.

- High-priority devices should obtain the bus quickly.

- Bus conflicts must be prevented.

Proposed Architecture

Peripheral 1 ─┐

Peripheral 2 ─┤

Peripheral 3 ─┤

...       ├──> Bus Arbiter ──> System Bus ──> CPU/Memory

Peripheral 12 ─┘

|

Priority Logic

Priority Classification

| Device Type | Priority |
| --- | --- |
| Emergency/Control devices | Highest |
| Sensors requiring fast response | High |
| Normal I/O devices | Medium |
| Logging/monitoring devices | Low |

Solution

A priority-based bus arbitration mechanism is implemented. Every peripheral sends a bus request when it needs to transfer data. The bus arbiter examines all requests and grants the bus to the highest-priority device.

For example:

P1 ─ Request ─┐

P2 ─ Request ─┤

P3 ─ Request ─┤

P4 ─ Request ─┤

v

+-------------+

| Bus Arbiter |

+-------------+

|

Highest Priority

|

v

Bus Grant

To prevent low-priority devices from being completely blocked, aging or time-limited priority can be implemented. If a low-priority device waits for a long time, its effective priority can gradually increase.

Justification

Priority-based arbitration gives control devices deterministic and fast bus access. Aging prevents starvation of low-priority devices. Therefore, the architecture provides both real-time response for critical devices and fair bus usage for other peripherals

- Construct a data transfer mechanism for a real-time audio processing system with the following constraints:

  - Audio data must be transferred continuously at 200 MB/s.

  - Transfer interruptions must be minimized.

  - Use double buffering for continuous data processing.

  - Generate an interrupt only when a buffer transfer is completed.

The audio processing system must continuously transfer data at 200 MB/s. Transfer interruptions must be minimized. Double buffering is required, and an interrupt should be generated only after a buffer transfer is completed.

Constraint Analysis

- Required continuous transfer rate = 200 MB/s.

- Data must not be interrupted.

- CPU should not receive an interrupt for every small transfer.

- Two buffers are required for continuous processing.

- Interrupt should occur only after completion of a buffer transfer.

Proposed Architecture

Audio Input

|

v

DMA Controller

/          \

v            v

+----------+   +----------+

| Buffer A |   | Buffer B |

+----------+   +----------+

|              |

v              v

Audio Processing Unit

Solution

A double-buffered DMA mechanism is used.

Initially, DMA fills Buffer A while the processor works on Buffer B. Once Buffer A becomes full, DMA generates a single interrupt indicating buffer completion. The processor then processes Buffer A while DMA starts filling Buffer B.

The operation continues alternately:

Time →

DMA:       [Buffer A] [Buffer B] [Buffer A] [Buffer B]

CPU:       [Buffer B] [Buffer A] [Buffer B] [Buffer A]

Interrupt:     ↑          ↑          ↑          ↑

Buffer      Buffer      Buffer      Buffer

complete    complete    complete    complete

Transfer Requirement

Required rate:

200 MB/s

For a 1 MB buffer:

Transfer time = 1 MB / 200 MB/s

= 0.005 s = 5 ms

Thus, approximately every 5 ms, a 1 MB buffer completes and generates one interrupt.

Justification

Double buffering allows data acquisition and processing to occur simultaneously. DMA handles continuous data movement, while the CPU receives only one interrupt per completed buffer instead of many interrupts for individual data transfers. This reduces interrupt overhead and supports continuous 200 MB/s audio processing.

- Design an interrupt management system for a multicore embedded processor with the following constraints:

  - Support multiple interrupt sources simultaneously.

  - Distribute interrupts across processor cores to balance the workload.

  - Critical interrupts must have higher priority than routine interrupts.

  - Prevent excessive interrupt overhead during high system activity.

The multicore embedded processor must handle multiple interrupt sources simultaneously. Interrupts should be distributed among processor cores to balance workload. Critical interrupts must have higher priority, and excessive interrupt overhead must be avoided during high system activity.

Constraint Analysis

- Multiple interrupt sources can occur simultaneously.

- Interrupt workload should be distributed across multiple cores.

- Critical events require immediate processing.

- Routine interrupts should have lower priority.

- Excessive interrupt handling can reduce CPU performance.

Proposed Architecture

Interrupt Sources

/   |    |    |    \

v    v    v    v     v

+---------------------------+

|    Interrupt Controller   |

+---------------------------+

|        |        |

v        v        v

Core 0   Core 1   Core 2

|        |        |

Critical   Network   Routine

Events     Events   Events

Interrupt Priority

Interrupts can be classified as:

| Interrupt Type | Priority |
| --- | --- |
| Emergency / Safety | Highest |
| Real-time control | High |
| Communication | Medium |
| Routine monitoring | Low |

The interrupt controller checks the priority of incoming interrupts and sends critical interrupts to the appropriate core immediately.

Load Balancing

Interrupts can be distributed among processor cores using an interrupt affinity mechanism.

For example:

Core 0 → Critical control interrupts

Core 1 → Communication interrupts

Core 2 → Sensor and routine interrupts

Core 3 → Background/system interrupts

If one core becomes heavily loaded, suitable interrupt sources can be redirected to another available core.

Reducing Interrupt Overhead

To prevent excessive interrupt overhead:

- Use DMA for high-volume data transfers.

- Use interrupt coalescing for frequent events.

- Group multiple completed operations into a single interrupt.

- Assign interrupt priorities.

- Distribute interrupt sources across cores.

Justification

The proposed interrupt management system ensures that critical events receive fast service while routine operations are distributed across available cores. DMA and interrupt coalescing reduce the number of interrupts generated during high activity. Therefore, the processor achieves better workload balancing, lower interrupt overhead, and improved real-time response.