# SIMATS ENGINEERING

### Department of Computer Science and Engineering

## ASSESSMENT TOOL 2 – PSEUDOCODE WRITING TASK

| Course Code: | CSA1220 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO5 – Precision (BL4) | Assessment: | Pseudocode Writing task |
| Weightage: | 30% | Total Marks: | 100 |
| CO5 Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: | Sanjay Kumar P | Reg. No.: | 192565073 |
| Section / Batch: | Slot B | Date: | 25-08-2026 |

Instructions to Students

- Identify the relevant concepts of DMA-based data transfer, vectored interrupts, I/O mapping techniques, bus arbitration, and hybrid I/O communication.

- Analyze the requirements of different I/O devices by considering CPU involvement, interrupt priority, latency, bandwidth, throughput, and transfer speed.

- Apply appropriate I/O techniques such as DMA, interrupt-driven I/O, vectored interrupts, memory-mapped I/O, I/O-mapped I/O, and bus arbitration to develop efficient solutions.

- Develop pseudocode algorithms for DMA data transfer, prioritized interrupt handling, dynamic I/O mapping selection, bus arbitration, and hybrid polling–DMA communication.

- Evaluate the proposed I/O mechanisms by interpreting CPU utilization, data transfer efficiency, interrupt response time, communication performance, resource allocation, and overall system suitability.

### QUESTIONS

- A real-time embedded system receives interrupt requests from sensors, communication modules, and storage devices at unpredictable intervals. Develop pseudocode for a nested interrupt handling mechanism that allows a higher-priority interrupt to pre-empt a lower-priority service routine. The algorithm should save the current processor context, service the higher-priority interrupt, restore the interrupted routine, and ensure that pending interrupts are handled correctly.

- A data-intensive embedded system must transfer a large amount of information between memory and multiple I/O devices without continuously occupying the processor. Write pseudocode for a scatter-gather DMA mechanism that transfers data stored in multiple non-contiguous memory locations. The algorithm should maintain a list of memory descriptors, initiate successive transfers, update descriptor status, and handle completion and error conditions.

- A computer system supports several I/O devices that generate data at different rates, causing temporary variations in the amount of data waiting for processing. Design pseudocode for an I/O buffering mechanism that manages incoming and outgoing data using appropriate buffer allocation and release strategies. The algorithm should monitor buffer occupancy, handle buffer-full and buffer-empty conditions, and prevent data loss while maintaining efficient device communication.

- A high-speed embedded platform contains multiple communication devices competing for shared system resources. Develop pseudocode for a priority-based resource allocation mechanism that assigns communication resources according to device urgency and bandwidth requirements. The algorithm should dynamically update priorities, allocate resources to eligible devices, release resources after transmission, and prevent low-priority devices from being indefinitely delayed.

- A real-time system must transfer data from several peripheral devices while meeting strict timing constraints. Write pseudocode for a real-time I/O scheduling mechanism that determines when each device should perform data transfers based on deadlines, transfer duration, priority, and available system bandwidth. The algorithm should schedule time-critical transfers first, monitor missed deadlines, and reschedule pending requests to improve overall system responsiveness.

### Code of Conduct

I sanjay kumar P(192565073)	(Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action.

## Signature of the Student: Sanjay Kumar P

### RUBRICS FOR CALCULATION

| Criteria | Weightage(%) | Level 4 – Exemplary (5 Marks) | Level 3 – Proficient (4 Marks) | Level 2 – Developing (2–3 Marks) | Level 1 – Beginning (0–1 Mark) |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Identifies all inputs, outputs, constraints accurately | Minor missing elements | Partial understanding | Incorrect interpretation |
| Algorithm Design | 30% | Complete, correct, and logically sound solution | Minor logical gaps | Partially correct logic | Incorrect approach |
| Use of Control Structures | 20% | Efficient and appropriate use of loops, conditions, functions/modules | Minor inefficiencies | Limited or basic usage | Incorrect or missing constructs |
| Pseudocode Structure | 10% | Well-structured, clear, consistent format, easy to follow | Mostly clear with minor issues | Difficult to follow in parts | Poorly structured and unclear |
| Completeness | 20% | Handles all cases; optimized approach | Handles most cases; reasonable efficiency | Handles basic cases only | Incomplete or inefficient |

## Marks Evaluation:

| Q. No | Problem Understanding (20) |  | Algorithm Design (30) |  | Use of Control Structures (20) | Pseudocode Structure (10) | Completeness (20) | Total (out of 100) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
| 2 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
| 3 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
| 4 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
| 5 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
|  | / 20 |  | /30 |  | / 20 | / 10 | / 20 | / 100 |

Course Coordinator	Faculty Incharge

- A real-time embedded system receives interrupt requests from sensors, communication modules, and storage devices at unpredictable intervals. Develop pseudocode for a nested interrupt handling mechanism that allows a higher-priority interrupt to pre-empt a lower-priority service routine. The algorithm should save the current processor context, service the higher-priority interrupt, restore the interrupted routine, and ensure that pending interrupts are handled correctly.

The system receives interrupts from sensors, communication modules, and storage devices at unpredictable intervals. A higher-priority interrupt must be able to pre-empt a lower-priority interrupt service routine. The processor context must be saved and restored correctly, and pending interrupts must be handled after the current interrupt is completed.

Pseudocode

ALGORITHM Nested_Interrupt_Handler

INPUT:

Interrupt requests from multiple devices

Priority of each interrupt

OUTPUT:

Correctly serviced interrupts

BEGIN

Initialize Interrupt_Controller

Initialize Pending_Interrupt_Queue

Enable Global_Interrupts

WHILE System_Is_Running DO

IF Interrupt_Request_Arrives THEN

Add interrupt to Pending_Interrupt_Queue

Sort Pending_Interrupt_Queue by Priority

IF New_Interrupt.Priority > Current_Interrupt.Priority THEN

Save Processor_Context()

Save Current_Interrupt_State()

Disable Lower_Priority_Interrupts

Current_Interrupt ← New_Interrupt

Service_Interrupt(Current_Interrupt)

Restore Processor_Context()

Restore Previous_Interrupt_State()

Enable Interrupts

END IF

END IF

WHILE Pending_Interrupt_Queue is NOT EMPTY DO

Next_Interrupt ← Highest_Priority(Pending_Interrupt_Queue)

IF Next_Interrupt is Higher_Or_Equal_Priority THEN

Remove Next_Interrupt from Queue

Save Processor_Context()

Service_Interrupt(Next_Interrupt)

Restore Processor_Context()

ELSE

BREAK

END IF

END WHILE

END WHILE

END

Explanation

When an interrupt arrives, its priority is checked. If it has a higher priority than the currently executing interrupt, the processor saves its current context and allows the higher-priority interrupt to execute. After completion, the saved context is restored and the interrupted routine continues.

Pending interrupts are stored in a priority queue so that the highest-priority pending interrupt is handled first.

Advantages

- Supports nested interrupts.

- Critical interrupts receive faster service.

- Processor context is safely preserved.

- Pending interrupts are not lost.

- Lower-priority routines can be resumed after pre-emption.

- A data-intensive embedded system must transfer a large amount of information between memory and multiple I/O devices without continuously occupying the processor. Write pseudocode for a scatter-gather DMA mechanism that transfers data stored in multiple non-contiguous memory locations. The algorithm should maintain a list of memory descriptors, initiate successive transfers, update descriptor status, and handle completion and error conditions.

The system must transfer large amounts of data between memory and multiple I/O devices without continuously occupying the processor. Data may exist in multiple non-contiguous memory locations. Therefore, a list of memory descriptors is maintained and processed sequentially.

Pseudocode

ALGORITHM Scatter_Gather_DMA

INPUT:

Descriptor_List

I/O_Device

Destination_Memory

OUTPUT:

Successfully transferred data

BEGIN

Initialize DMA_Controller

Initialize Descriptor_List

FOR each Descriptor IN Descriptor_List DO

IF Descriptor.Status = INVALID THEN

Report_Error("Invalid Descriptor")

CONTINUE

END IF

Source_Address ← Descriptor.Source

Destination_Address ← Descriptor.Destination

Transfer_Size ← Descriptor.Size

Configure_DMA(

Source_Address,

Destination_Address,

Transfer_Size

)

Descriptor.Status ← IN_PROGRESS

Start_DMA_Transfer()

WAIT until DMA_Transfer_Complete OR DMA_Error

IF DMA_Error THEN

Descriptor.Status ← ERROR

Record_Error(Descriptor)

IF Error_Is_Critical THEN

Stop_DMA()

RETURN FAILURE

ELSE

CONTINUE

END IF

ELSE

Descriptor.Status ← COMPLETED

Update_Transferred_Bytes(Transfer_Size)

END IF

END FOR

IF All_Descriptors_Completed THEN

Generate_DMA_Completion_Interrupt()

RETURN SUCCESS

END IF

END

Example Descriptor List

Descriptor 1 → Memory Block A → Transfer

Descriptor 2 → Memory Block C → Transfer

Descriptor 3 → Memory Block F → Transfer

Descriptor 4 → Memory Block H → Transfer

The memory blocks do not need to be physically adjacent.

Explanation

Scatter-gather DMA allows the DMA controller to collect data from multiple non-contiguous memory locations and transfer it to an I/O device. Each descriptor contains the source address, destination address, transfer size, and status.

The CPU only initializes the descriptor list and handles completion or serious errors. This greatly reduces CPU involvement.

Advantages

- Supports non-contiguous memory.

- Reduces CPU overhead.

- Efficient for large data transfers.

- Supports multiple memory blocks.

- Provides error and completion handling.

- A computer system supports several I/O devices that generate data at different rates, causing temporary variations in the amount of data waiting for processing. Design pseudocode for an I/O buffering mechanism that manages incoming and outgoing data using appropriate buffer allocation and release strategies. The algorithm should monitor buffer occupancy, handle buffer-full and buffer-empty conditions, and prevent data loss while maintaining efficient device communication.

Different I/O devices generate data at different rates. Temporary increases in incoming data can cause buffers to become full, while slow data production can cause buffers to become empty. The buffering mechanism must prevent data loss and maintain efficient communication.

Pseudocode

ALGORITHM IO_Buffer_Manager

INPUT:

Incoming_Data

Outgoing_Data

Buffer_Size

OUTPUT:

Efficient data transfer without data loss

BEGIN

Create Circular_Buffer(Buffer_Size)

WHILE System_Is_Running DO

IF Incoming_Data_Available THEN

IF Buffer is NOT FULL THEN

Data ← Read_From_Device()

Write_To_Buffer(Data)

Update_Buffer_Occupancy()

ELSE

IF Overflow_Buffer_Available THEN

Store_Data_In_Overflow_Buffer()

ELSE

Generate_Flow_Control_Signal()

Temporarily_Stop_Input()

END IF

END IF

END IF

IF Output_Device_Ready THEN

IF Buffer is NOT EMPTY THEN

Data ← Read_From_Buffer()

Send_To_Output_Device(Data)

Update_Buffer_Occupancy()

ELSE

Generate_Underflow_Warning()

Wait_For_New_Data()

END IF

END IF

IF Buffer_Occupancy > HIGH_WATERMARK THEN

Enable_Flow_Control()

ELSE IF Buffer_Occupancy < LOW_WATERMARK THEN

Allow_More_Input()

END IF

END WHILE

END

Buffer Structure

Circular Buffer

+----+----+----+----+----+----+

| D1 | D2 | D3 |    |    |    |

+----+----+----+----+----+----+

↑                 ↑

Read Pointer      Write Pointer

Explanation

A circular buffer is used to efficiently manage incoming and outgoing data. When the buffer is not full, new data is stored. If the buffer becomes full, flow control or an overflow buffer prevents data loss. When the output device is ready, data is removed from the buffer.

High and low watermarks help control the input rate based on buffer occupancy.

Advantages

- Prevents buffer overflow.

- Handles different device data rates.

- Prevents unnecessary data loss.

- Supports continuous communication.

- Uses memory efficiently.

- A high-speed embedded platform contains multiple communication devices competing for shared system resources. Develop pseudocode for a priority-based resource allocation mechanism that assigns communication resources according to device urgency and bandwidth requirements. The algorithm should dynamically update priorities, allocate resources to eligible devices, release resources after transmission, and prevent low-priority devices from being indefinitely delayed.

Multiple communication devices compete for shared system resources. Resources must be assigned based on device urgency and bandwidth requirements. Priorities should change dynamically, and low-priority devices must not be indefinitely delayed.

Pseudocode

ALGORITHM Priority_Based_Resource_Allocation

INPUT:

Device_Request_List

Available_Bandwidth

Available_Resources

OUTPUT:

Allocated communication resources

BEGIN

Initialize Device_Request_List

WHILE System_Is_Running DO

FOR each Device IN Device_Request_List DO

IF Device.Request = TRUE THEN

Calculate_Priority(Device)

IF Device.Waiting_Time > MAX_WAIT_TIME THEN

Increase_Priority(Device)

END IF

IF Device.Deadline is Near THEN

Increase_Priority(Device)

END IF

END IF

END FOR

Sort Device_Request_List by Priority DESCENDING

FOR each Device IN Device_Request_List DO

IF Device.Request = TRUE THEN

IF Required_Bandwidth(Device)

<= Available_Bandwidth THEN

Allocate_Resource(Device)

Available_Bandwidth ←

Available_Bandwidth -

Device.Required_Bandwidth

Start_Transmission(Device)

WAIT until Transmission_Complete

Release_Resource(Device)

Available_Bandwidth ←

Available_Bandwidth +

Device.Required_Bandwidth

END IF

END IF

END FOR

END WHILE

END

Priority Calculation

Priority =

Base_Priority

+ Urgency

+ Deadline_Factor

+ Waiting_Time_Factor

Explanation

The system continuously monitors communication requests. Each device receives a priority based on urgency, bandwidth requirement, deadline, and waiting time.

Aging is used to increase the priority of devices that have been waiting for a long time. This prevents starvation of low-priority devices.

Advantages

- Critical devices receive resources first.

- Bandwidth is allocated according to requirements.

- Priorities can change dynamically.

- Low-priority devices are prevented from starvation.

- Resources are released after transmission.

- A real-time system must transfer data from several peripheral devices while meeting strict timing constraints. Write pseudocode for a real-time I/O scheduling mechanism that determines when each device should perform data transfers based on deadlines, transfer duration, priority, and available system bandwidth. The algorithm should schedule time-critical transfers first, monitor missed deadlines, and reschedule pending requests to improve overall system responsiveness.

Several peripheral devices need data transfers under strict timing constraints. The scheduler must consider deadlines, transfer duration, priority, and available bandwidth. Time-critical transfers must be scheduled first, while missed requests should be monitored and rescheduled.

Pseudocode

ALGORITHM Real_Time_IO_Scheduler

INPUT:

I/O_Request_List

Available_Bandwidth

Current_Time

OUTPUT:

Scheduled I/O transfers

BEGIN

Initialize Ready_Queue

Initialize Missed_Deadline_List

WHILE System_Is_Running DO

FOR each Request IN I/O_Request_List DO

IF Request.Arrival_Time <= Current_Time

AND Request.Status = PENDING THEN

Calculate_Time_To_Deadline(Request)

Add Request to Ready_Queue

END IF

END FOR

FOR each Request IN Ready_Queue DO

IF Current_Time > Request.Deadline THEN

Request.Status ← MISSED

Add Request to Missed_Deadline_List

END IF

END FOR

Sort Ready_Queue using:

1. Earliest Deadline

2. Higher Priority

3. Shorter Transfer Duration

FOR each Request IN Ready_Queue DO

IF Required_Bandwidth(Request)

<= Available_Bandwidth THEN

IF Current_Time + Request.Transfer_Time

<= Request.Deadline THEN

Allocate_Bandwidth(Request)

Start_IO_Transfer(Request)

WAIT until Transfer_Complete

Release_Bandwidth(Request)

Request.Status ← COMPLETED

Remove Request from Ready_Queue

END IF

END IF

END FOR

FOR each Request IN Missed_Deadline_List DO

IF Request.Can_Be_Rescheduled THEN

Update_Deadline(Request)

Update_Priority(Request)

Request.Status ← PENDING

ELSE

Report_Missed_Deadline(Request)

END IF

END FOR

Current_Time ← Update_System_Time()

END WHILE

END

Scheduling Logic

I/O Requests

|

v

Check Deadlines

|

v

Ready Queue

|

v

+--------------------+

| Priority Scheduler |

+--------------------+

|

+-----+-----+

|           |

Time-Critical   Normal

|           |

+-----+-----+

|

v

I/O Transfer

|

v

Resource Release

Explanation

The scheduler first checks the arrival time and deadline of every request. Requests that are ready are placed in the ready queue. The queue is prioritized based on earliest deadline, priority, and transfer duration.

Before starting a transfer, the scheduler checks whether enough bandwidth is available and whether the transfer can finish before its deadline.

If a deadline is missed, the request is recorded and rescheduled when possible. This improves overall system responsiveness.

Advantages

- Time-critical transfers are handled first.

- Deadlines are continuously monitored.

- Bandwidth is allocated efficiently.

- Missed requests can be rescheduled.

- Prevents unnecessary resource conflicts.

- Improves real-time I/O performance.